# CIS179 Chapter 1 Lecture notes

# Why study Networking: Statistics (Video: Why Study Networking Part 1: Statistics 3:05)

In 2019 there are about 4.3 billion users online about 56% of the world’s population (of 7.6 billion population)

3.5 billion are using social media

3.4 billion of them are using social media on their phones

50% of internet users are in the Asian continent

7% of Internet users are in North America

89.4% of the US population are on the Internet

80% of Americans have made an online purchase this month

96% of Americans have made purchases online in their lifetime.

40% of all purchases in America are made online

44% of purchases in America are at Amazon

# Why study networkingPart II : Jobs (Video: Why study networking Part II: Jobs 4:32)

# Terminology (Video: Terminology 4:44)

Data network: A transmission system that enables computer networking

Computer network: two or more computers or computer-enabled devices connected such that they can share data and resources.

Figure 1 Business network

Figure 2 Home network

Other networks exist as well: POTS, Cell phone, electric grid, mail system, radio, broadcast TV

Convergence: When two technologies overlap

Examples: Cell system/Internet, TV/Internet, firewalls/content filters

# Milestones (Video: Milestones in communication 6:39)

- Telegraph: Samuel Morse 1880, first remote distance communication

  - Demonstrates the three parts of communication, Information, Code, Signal

- Multiplexing: Combining multiple signals on one medium

  - Reduces cost (cabling is expensive)

- Switching: The ability to direct traffic to a desired destination

- Packetization: Ability to utilize more of the capability of the medium

  - Requires that messages be broken into segments at the sender end

  - Packetization reduces burstiness

# Layering (Video: Layering 3:50)

Layering is breaking down the communication process into steps to accomplish several tasks

  - User information and commands (the input and output)

  - Segmentation and reassembly

  - Identifying source and destination

  - Check for errors and correct them

  - Signaling

These must account for both local and distance communication

- Switches handle local addressing

- Routers handle distance addressing

  - Hop: The transfer of data from one router to another is one hop

# OSI Model (Video: OSI model Intro 3:14)

7. Application layer

6. Presentation Layer

5. Session layer

4. Transport layer

3. Network Layer

2. Data-Link Layer

1. Physical layer

- Standardization

- Interaction with only the next-door neighbor

# TCI/IP Protocol suite (stack) (Video: Protocol suites 7:25)

- Protocol: A set of rules by which communication is performed

- Several are used during any one exchange of data – simultaneous use of multiple protocols.

- TCP/IP is a set of many protocols that are compatible with each other

- Each protocol communicates with those protocols at the neighboring layer

# TCP/IP Model (Video TCP/IP model 4:08)

- Better fit with the TCI/IP protocol suite

- Application layer

- Transport layer

- Internetwork layer

- Network Access layer (sometimes called the Link layer)

The OSI model layers are referred to by number. Do not do that in the TCP/IP model.

P 23 Table 1.4 Network layer tasks (Video: Encapsulation and layer tasks 3:32)

| Layer Number/Name | Task | Header/footer information |
| --- | --- | --- |
| 7, 6, 5 Application | User data/commands | User commands |
| 4 Transport | Segmentation/reassembly | Sequence numbers, port numbers |
| 3 Network | Distance addressing | IP address |
| 2 Data link | Local addressing | Local address/Error check (footer) |
| 1 Physical | Signaling | None |

P24 Table 1.14 Protocols (Video: Common Protocols 3:12)

| Layer | Protocols |
| --- | --- |
| 7, 6, 5 Application | HTTP, SMTP, IMAP, DHCP, DNS, FTP |
| 4 Transport | TCP, UDP |
| 3 Network | IP |
| 2 Data-Link | Ethernet, Frame Relay, ATM |
| 1 Physical | AM, FM, Fiber, SONET, CDMA |

Note the hourglass shape of the protocols. IP is central to all communication

P25 Table 1.15 Layer functions (Video: What the OSI layers do 4:33)

| OSI layer | Layer function |
| --- | --- |
| 7. Application | Data generation |
| 6. Presentation | Coding. Preparation of data to the form in which it will be transmitted |
| 5. Session | Create and terminate sessions |
| 4. Transport | Segmentation, multiplexing |
| 3. Network | IP addressing |
| 2. Data-link | Error checking, local addressing |
| 1. Physical | Convert from code to signals |

Mnemonics

All people seem to need data processing

Programmers do not throw sausage pizza away

Table of Protocol Data Units (PDU) (Video: OSI PDUs 2:01)

| OSI layer | Layer function |
| --- | --- |
| 7. Application | Data |
| 6. Presentation | Data |
| 5. Session | Data |
| 4. Transport | Segment |
| 3. Network | Packet |
| 2. Data-link | Frame |
| 1. Physical | Bit |

Table of equipment (Video: OSI Devices 1:14)

| OSI Layer | Equipment |
| --- | --- |
| 7, 6, 5 Application | End-user equipment (hosts) |
| 4 Transport | End-user equipment (hosts) |
| 3 Network | Routers (perform routing across networks) |
| 2 Data-Link | Switches (local communication) |
| 1 Physical | Wire, Radio signals, Fiberoptic, NIC, Hub, Repeater |